# SYDNEY C Roofing

## Design Structural Analysis

## Order number 	 20/stat.15	2020-07-29

Author: ing. V. Chobot, Tábor, Buzulucká 2332

Authorized Civil Engineer, ČKAIT 0101501

## Complied for: ALBIXON a.s. Cintlovka 535, 268 01 Hořovice

Volume content:

Technical report	pp. 2-3

Drawings of the structure 1 and 2	pp. 4-5

Calculation model	p. 6

Structural analysis of KZS 1	pp. 7-10

Structural analysis of KZS 2	pp. 11-14

Conclusion	pp. 15

Technical report:

Assessment of the stability of the “SYDNEY C 5000 x 2190 mm” skeleton roof structure made of aluminium profiles according to EN AW 6063. The roof will be installed on the outside. Climate load specified by the customer: Snow of 0.6 kNm-2, wind with a reference pressure corresponding to 120 km/h. The shape of the skeleton structure is specified according to the drawing attached. The structure will be provided with a lightweight roof cladding made of organic glass panels with an adjustable shape, 4 – 6 mm, or polycarbonate honey-comb panels. The structure will be anchored to a travel rail in the bases of the arches.

Assessed loading conditions:

ZS 1. Self-load weight of 0.111 t, including roofing, for a span of 5 m.

ZS 2. Vertical surface snow load of 0.60 kNm-2.

ZS 3. Horizontal roof surface wind load ( vref = 33.33 ms-1; γair= 1.25 e-3 tm-3; ; q= 7.5 kNm-2; suction of 3.75kNm-2).

Computation load combinations:

Combination of roof load KZS 1 = 1.1 x ZS 1 + ZS 2; snow load of 0.6 kNm-2.

Combination of roof load KZS 2 = 1.1 x ZS 1 + ZS 3; super position of wind load of 33.33 kNm-1.

Standard references:

Publication date: March 1, 2010

Execution of steel structures and aluminium structures – Part 1: Requirements for conformity assessment of structural components.

EN 1090-1:2010/Z1 (732601), Date of issue: September 1, 2010 *Amendment

Execution of steel structures and aluminium structures – Part 1: Requirements for conformity assessment of structural components.

EN 1090-3 (732601), Date of issue: March 1, 2009

Execution of steel structures and aluminium structures – Part 3: Technical requirements for aluminium structures.

EN 1991-1-3 (730035), Date of issue: June 1, 2005

Eurocode 1: Loads on structures - Part 1-3: General loads - Snow loads.

EN 1991-1-3:2005/Correction 1 (730035), Date of issue: February 1, 2010 *Correction

Eurocode 1: Actions on structures - Part 1-3: General actions - Snow loads.

EN 1991-1-3:2005/Z1 (730035), Date of issue: October 1, 2006 *Amendment

Eurocode 1: Actions on structures - Part 1-3: General actions - Snow loads.

EN 1991-1-3:2005/Z2 (730035), Date of issue: February 1, 2010 *Amendment

Eurocode 1: Actions on structures - Part 1-3: General actions - Snow loads.

EN 1991-1-3:2005/Z3 (730035), Date of issue: March 1, 2010 *Amendment

Eurocode 1: Actions on structures - Part 1-3: General actions - Snow loads.

EN 1991-1-4 (730035), Date of issue: April 1, 2007

Eurocode 1: Loads on structures - Part 1-4: General loads - Wind loads.

EN 1991-1-4:2007/A1 (730035), Date of issue: October 1, 2010 *Amendment

Eurocode 1: Actions on structures - Part 1-4: General loads - Wind loads.

EN 1991-1-4:2007/Correction 1 (730035), Date of issue: September 1, 2008 *Correction

Eurocode 1: Actions on structures - Part 1-4: General loads - Wind loads.

EN 1991-1-4:2007/Correction 2 (730035), Date of issue: May 1, 2010 *Correction

Eurocode 1: Actions on structures - Part 1-4: General loads - Wind loads.

EN 1991-1-4:2007/Correction 3 (730035), Date of issue: January 1, 2011 *Correction

Eurocode 1: Actions on structures - Part 1-4: General loads - Wind loads.

EN 1991-1-4:2007/Z1 (730035), Date of issue: March 1, 2010 *Amendment

Eurocode 1: Actions on structures - Part 1-4: General loads - Wind loads.

List of specified and used materials:

| E1, E2 | [MPa] | Modules of elasticity (E2 only for ortho-tropic material) |
| --- | --- | --- |
| ni |  | Poisson effect |
| Gamma | [t/m3] | Bulk density |
| K1, K2 | [kN/m3] | Coefficients of thermal expansion |
| Attenuation |  | Decrement of attenuation |

| Material | Type | E 1 | ni | Gamma | K 1 | E 2 | K 2 | Attenuation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [MPa] |  | [t/m3] | [kN/m3] | [MPa] | [kN/m3] |  |
| Aluminium | STEEL | 72000.000 | 0.330 | 2.700 | 2.340e-05 |  |  | 0.010 |
| PC 1000 | OTHER | 2400.000 | 0.360 | 1.200 | 6.500e-06 |  |  |  |

| Material | Volume | Weight |
| --- | --- | --- |
|  | [m3] | [t] |
| Aluminium | 0.023 | 0.063 |
| PC 1000 | 0.040 | 0.048 |
| Total |  | 0.111 |

List of cross-sections:

| Iy, Iz | [mm4] | Main moments of inertia |
| --- | --- | --- |
| Ik | [mm4] | Moment of stiffness in simple twist |
| Beta y, beta z |  | Shear stiffness coefficients |
| P |  | Full cross section |
| S |  | Combined |
| D |  | Partial |

| Cross section | Type | Material | Area | ly | Iz | Ik | beta y | beta z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | [m2] | [m4] | [m4] | [m4] |  |  |
| 01-006 | S |  | 5.465e-04 | 1.066e-07 | 1.527e-07 | 8.599e-09 | 0.590 | 0.599 |
| - External outline | D | Aluminium | 5.465e-04 | 1.066e-07 | 1.527e-07 | 8.599e-09 | 0.590 | 0.599 |
| 03-003 | S |  | 1.310e-04 | 1.598e-08 | 7.640e-09 | 3.122e-10 | 0.482 | 0.525 |
| - External outline | D | Aluminium | 1.310e-04 | 1.598e-08 | 7.640e-09 | 3.122e-10 | 0.482 | 0.525 |
| - Hole No.1 | D | Aluminium | 1.310e-04 | 1.598e-08 | 7.640e-09 | 3.122e-10 | 0.482 | 0.525 |
| 03-001 | S |  | 3.277e-04 | 5.758e-08 | 4.807e-08 | 2.728e-09 | 0.711 | 0.415 |
| - External outline | D | Aluminium | 3.277e-04 | 5.758e-08 | 4.807e-08 | 2.728e-09 | 0.711 | 0.415 |
| 01-008 | S |  | 8.620e-04 | 1.813e-07 | 3.703e-07 | 2.502e-08 | 0.734 | 0.390 |
| - External outline | D | Aluminium | 8.620e-04 | 1.813e-07 | 3.703e-07 | 2.502e-08 | 0.734 | 0.390 |
| 01-028 | S |  | 2.869e-03 | 2.789e-05 | 1.017e-06 | 5.861e-08 | 0.532 | 0.760 |
| - External outline | D | Aluminium | 2.869e-03 | 2.789e-05 | 1.017e-06 | 5.861e-08 | 0.532 | 0.760 |

Units used:

| Geometry - length | mm | Deformation - displacements | mm |
| --- | --- | --- | --- |
| Geometry - angles | deg | Deformation - rotation | deg |
| Cross-sections - length | mm | Time | s |
| Loading, results - forces | kN | Temperature | °C |
| Loading, results - voltage | MPa | Mass | t |
| Loading, results - length | mm |  |  |

Comparative calculation values for material according to EN AW 6063 (AlMgSi)

The structure significance for the service life is 25 years according to EN 1991-1.

Minimum tensile strength	245 MPa

Minimum yield strength Rp 0.2	200MPa

Derived comparative tension for EN AW 6063 σ = 142.3 MPa

Derived comparative tension for PC 1000 polycarbonate σ = 142 MPa

| ROZMÍSTĚNÍ VZPĚR (PROFIL 03-003) A DĚLÍCÍHO PROFILU (PROFIL 03-001) | LAYOUT OF STRUTS (PROFILE 03-003) AND DIVIDING PROFILE (PROFILE 03-001) |
| --- | --- |
| PROFIL | PROFILE |
| ČEP | PIN |
| NEREZ | STAINLESS STEEL |
| ŠROUB | SCREW |
| POPIS POSLEDNÍ ZMĚNY INDEXU | DESCRIPTION OF LATEST INDEX CHANGE |
| TOLEROVANÍ ISO 8015 | TOLERANCE ISO 8015 |
| VŠEOBECNÉ TOLERANCE ISO 2768 mK | GENERAL TOLERANCES ISO 2768 mK |
| KRESLIL | DRAWN BY |
| DATUM | DATE |
| NÁZEV 1 | TITLE 1 |
| MATERIÁL | MATERIAL |
| POLOTOVAR | INTERMEDIATE |
| HMOTNOST (kg) | WEIGHT (kg) |
| NÁZEV 2 | TITLE 2 |
| FORMAT | FORMAT |
| MĚŘÍTKO | SCALE |
| VÝKRES | DRAWING |
| INDEX | INDEX |
| LIST | SHEET |

| ROZMÍSTĚNÍ VZPĚR (PROFIL 03-003) A DĚLÍCÍHO PROFILU (PROFIL 03-001) | LAYOUT OF STRUTS (PROFILE 03-003) AND DIVIDING PROFILE (PROFILE 03-001) |
| --- | --- |
| PROFIL | PROFILE |
| ČEP | PIN |
| NEREZ | STAINLESS STEEL |
| ŠROUB | SCREW |
| POPIS POSLEDNÍ ZMĚNY INDEXU | DESCRIPTION OF LATEST INDEX CHANGE |
| TOLEROVANÍ ISO 8015 | TOLERANCE ISO 8015 |
| VŠEOBECNÉ TOLERANCE ISO 2768 mK | GENERAL TOLERANCES ISO 2768 mK |
| KRESLIL | DRAWN BY |
| DATUM | DATE |
| NÁZEV 1 | TITLE 1 |
| MATERIÁL | MATERIAL |
| POLOTOVAR | INTERMEDIATE |
| HMOTNOST (kg) | WEIGHT (kg) |
| NÁZEV 2 | TITLE 2 |
| FORMAT | FORMAT |
| MĚŘÍTKO | SCALE |
| VÝKRES | DRAWING |
| INDEX | INDEX |
| LIST | SHEET |

Rod elements

| Rod | Rod |
| --- | --- |

Structure assessment Roof loading by snow KZS 1 = 1.1 x ZS 1 + ZS 2

The results of the entire structure assessment - internal forces, all rods, total limits for the module

Derived comparative tension for EN AW 6063 σ = 142.3 MPa

Derived comparative tension for PC 1000 polycarbonate σ = 142 MPa

Loading scheme KZS 1:

## Results of the assessment - internal forces, all rods, selected results, total limits

Sig.min, Sig.max	[MPa]	Tension on outer fibres

Limits for result: KZS1 - snow

| Rod | Position | Sig.min | Sig.max |
| --- | --- | --- | --- |
|  | [m] | [MPa] | [MPa] |
| Rod478 | 1.000 | -27.438 | 31.633 |
| Rod482 | 0.500 | -0.014 | 0.011 |
| Rod451 | 0.000 | -10.859 | -9.674 |
| Rod478 | 1.000 | -27.438 | 31.633 |

## The design is compliant, the comparative tension is not reached.

## Results of the assessment - deformation, all rods, selected results, total limits

Ux, Uy, Uz	[mm]	displacements in axes

Utotal	[mm]	total displacements

Limits for result: KZS1 snow

| Rod | Position | Ux | Uy | Uz | Utotal |
| --- | --- | --- | --- | --- | --- |
|  | [m] | [mm] | [mm] | [mm] | [mm] |
| Rod475 | 1.500 | -0.331 | 0.010 | -2.742 | 2.762 |
| Rod478 | 0.500 | 0.331 | -0.010 | -2.741 | 2.760 |
| Ro461 | 0.352 | -0.098 | -0.048 | -1.205 | 1.209 |
| Rod464 | 0.352 | 0.097 | 0.048 | -1.202 | 1.207 |
| Rod477 | 0.500 | 0.265 | -0.011 | -3.522 | 3.532 |
| Rod477 | 0.500 | 0.265 | -0.011 | -3.522 | 3.532 |

## The ratio of max. deflection and assessed structure span of 5000 mm is 1: 1415

The curve of tension on the structure from KZS 1:

The curve of deformation on the structure from KZS 1:

Results of the assessment - internal forces, all surfaces, panels

sx, sy, sxy, sef	[MPa]	Tension in local axes

Derived comparative tension for PC 1000 4 mm polycarbonate panels σ = 142 MPa

Limits for result: KZS1 snow

| Area | Node | Position | sef | sef | sef |
| --- | --- | --- | --- | --- | --- |
|  |  |  | top | median | bottom |
|  |  | [m] | [MPa] | [MPa] | [MPa] |
| Polygon22 | 110 | 2.478, 1.000, 0.231 | 0.087 | 0.073 | 0.067 |
| Polygon25 | 63 | 1.328, -0.500, 0.181 | 7.772 | 0.665 | 8.999 |
| Polygon22 | 110 | 2.478, 1.000, 0.231 | 0.087 | 0.073 | 0.067 |
| Polygon25 | 63 | 1.328, -0.500, 0.181 | 7.772 | 0.665 | 8.999 |
| Polygon22 | 110 | 2.478, 1.000, 0.231 | 0.087 | 0.073 | 0.067 |
| Polygon25 | 63 | 1.328, -0.500, 0.181 | 7.772 | 0.665 | 8.999 |

### Isolines of tension in panels:

## Results of the assessment - transformation, all surfaces, panels

Ux, Uy, Uz	[mm]	displacements in axes

Utotal	[mm]	total displacements

Limits for result: KZS1 snow

| Area | Node | Position | Uz | Utotal |
| --- | --- | --- | --- | --- |
|  |  | [m] | [mm] | [mm] |
| Polygon21 | 159 | 3.170, 0.354, 0.209 | -28.317 | 28.367 |
| Polygon21 | 159 | 3.170, 0.354, 0.209 | -28.317 | 28.367 |

### Isolines of transformation in panels:

| Def.celk[mm] | Def. total [mm] |
| --- | --- |

## Structure assessment Roof loading by load superposition KZS 2 = 1.1 x ZS 1 + ZS 3

## The results of the entire structure assessment - internal forces, all rods, total limits for the module

### Derived comparative voltage for calculated effective values for Al σ = 142.3 MPa

### Loading scheme KZS 3:

## Results of the assessment - internal forces, all rods, selected results, total limits

Sig.min, Sig.max	[MPa]	Tension on outer fibres

Derived comparative tension for PC 1000 4 mm polycarbonate panels σ = 142.3 MPa

Limits for result: KZS2 - loading superposition

| Rod | Position | Sig.min | Sig.max |
| --- | --- | --- | --- |
|  | [m] | [MPa] | [MPa] |
| Rod453 | 0.352 | -120.183 | 87.525 |
| Rod454 | 0.636 | 7.437 | 9.362 |
| Rod1 | 0.000 | -10.302 | -10.029 |
| Rod449 | 0.000 | -78.272 | 100.523 |

## Design complies, comparative tension not exceeded - compliant.

## Results of the assessment - deformation, all rods, selected results, total limits

Ux, Uy, Uz	[mm]	displacements in axes

Utotal	[mm]	total displacements

Limits for result: KZS2 - loading superposition

| Rod | Position | Ux | Uy | Uz | Utotal |
| --- | --- | --- | --- | --- | --- |
|  | [m] | [mm] | [mm] | [mm] | [mm] |
| Rod474 | 1.500 | -5.683 | -0.010 | -36.179 | 36.623 |
| Rod469 | 0.704 | -4.186 | -0.055 | -25.172 | 25.518 |
| Ro461 | 0.704 | -4.184 | 0.060 | -25.161 | 25.506 |
| Rod474 | 1.500 | -5.683 | -0.010 | -36.179 | 36.623 |
| Rod477 | 1.500 | -5.191 | 0.017 | 33.351 | 33.753 |
| Rod474 | 1.500 | -5.683 | -0.010 | -36.179 | 36.623 |

### The ratio of max. deflection and assessed structure span of 5000 mm is 1: 136

The curve of tension on the structure from KZS 2:

The curve of deformation on the structure from KZS 2:

## Results of the assessment - internal forces, all surfaces, polycarbonate panels

sx, sy, sxy, sef	[MPa]	Tension in local axes

Derived comparative tension for PC 1000 4 mm polycarbonate panels σ = 142 MPa

Limits for result: KZS2 - loading superposition

| Area | Node | Position | sef | sef | sef |
| --- | --- | --- | --- | --- | --- |
|  |  |  | top | centerline | bottom |
|  |  | [m] | [MPa] | [MPa] | [MPa] |
| Polygon27 | 42 | 0.628, 0.000, 0.102 | 0.059 | 0.196 | 0.426 |
| Polygon20 | 232 | 4.328, 0.500, 0.102 | 12.151 | 0.455 | 12.991 |
| Polygon25 | 112 | 2.028, 0.000, 0.224 | 0.467 | 0.045 | 0.405 |
| Polygon22 | 145 | 2.928, -0.500, 0.224 | 2.027 | 1.353 | 4.581 |
| Polygon18 | 245 | 4.956, 0.500, 0.000 | 0.730 | 0.322 | 0.120 |
| Polygon20 | 232 | 4.328, 0.500, 0.102 | 12.151 | 0.455 | 12.991 |

### Isolines of tension in panels from KZS 2:

## Results of the assessment - deformation, all rods, selected results, total limits

| Ux, Uy, Uz |  | [mm] | displacements in axes |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Utotal |  | [mm] | total displacements |  |  |  |  |
| Limits for result: KZS2 |  |  | by snow |  |  |  |  |
|  | Area | Node | Position | Ux | Uy | Uz | Utotal |
|  |  |  | [m] | [mm] | [mm] | [mm] | [mm] |
|  | Polygon20 | 194 | 3.870, 0.354, 0.154 | -9.549 | -8.516e-03 | -70.421 | 71.065 |
|  | Polygon18 | 202 | 4.328, 1.000, 0.102 | -4.186 | -0.055 | -25.172 | 25.518 |
|  | Polygon17 | 210 | 4.328, -1.000, 0.102 | -4.184 | 0.060 | -25.161 | 25.506 |
|  | Polygon20 | 194 | 3.870, 0.354, 0.154 | -9.549 | -8.516e-03 | -70.421 | 71.065 |
|  | Polygon26 | 72 | 1.087, 0.353, 0.154 | -6.760 | 0.012 | 47.428 | 47.907 |
|  | Polygon20 | 194 | 3.870, 0.354, 0.154 | -9.549 | -8.516e-03 | -70.421 | 71.065 |

The ratio of max. deflection and assessed structure span of 5000 mm is 1: 71

### Isolines of transformation in panels from KZS 2:

| Def.celk[mm] | Def. total [mm] |
| --- | --- |

## Conclusion:

The assessment has shown that the SYDNEY C roof structure with the span of 5000 x 2190 mm is compliant for a snow load of 0.60 kNm-2, in combination 1, with a self-load of 0.111 t. It is also compliant for wind snow, for a reference speed of 120 km/h-1 (3.33 m.s-1) in the above superpositions KZS 1; KZS 2. It has been shown that the designed roof structure meets the mechanical requirements for the limit strength, usability limit and planned life of 2.19*105 hours according to EN 1991-1 and additional standards, and decrees specifying technical requirements for structures 268/09 Coll. as amended by Decree 20/12 Coll. Section 9 Mechanical resistance and stability.

It has been proven by the assessment that the structure cannot collapse, as neither of its parts reaches the limits of comparative tension.

There is no unacceptable transformation of the structure, as was demonstrated by the previous assessment.

There is no risk of damage to other parts of the building, or its technical facilities and installed equipment, due to unacceptable transformation.

Damage, in case of disproportionate extent to cause, is eliminated by the calculated degree of safety of Sb 2 for the main structures.